REPORT ON HARDWARE COMPONENTS OF A HUMANOID ROBOT

A Humanoid  Robot  is designed to Look Like a Human Body in structure and Movement and to perform the activities performed by humans. To interact with humans and its surroundings it needs three major abilities, They are Sensing, Processing and Actuation.

Sensing: Collecting information about the environment and its own body.

Processing: Analyzing  the collected data from sensor and making decisions.

Actuation: Converting commands into physical movements.

Some Major Hardware Components:

- RADAR(Radio Detection and Ranging) :

It is an object-detection system that uses radio waves to determine the distane, direction, and speed of objects.

USES :

- Detects nearby and moving objects.

- Estimates distance and relative speed.

- Supports obstacle detection in low-light conditions.

STRENGTHS : It can operate in foggy, dust and rainy conditions.

LIMITATIONS: May Struggle to dinstinguish Closely Spaced Objects

- LiDAR(Light Detection and Ranging):

LiDAR uses laser light to measure distances and build a 3-D representation of the surroundings. It emits laser pulses and measures their return time.

USES:

- Creates 3D maps of rooms and surroundings.

- Measures distances to obstacles.

- Supports navigation and collision avoidance.

- Helps estimate the position of objects in space.

STRENGTHS: It Offers unmatched precision, speed and environmental Flexibility.

LIMITATIONS: Performance may degrade in heavy rain, fog, or airborne dust.

- Inertial Measurement Units(IMUs):

An IMU measures the Robot’s Movements using acceleromotors and gyroscopes. It also maintains balance in humanoids.

USES :

- Helps maintain balance during walking.

- Detects sudden movements, tilting, and falls.

- Supports stabilization of the torso and head.

STRENGTHS: IMUs sample data at incredibly fast rates.

LMITATIONS: Vibrations and sudden impacts can affect measurements.

- Encoders : Encoders measure the position or rotation of a motor or joint. Some also provide speed information.

USES:

- Provides feedback for correcting movement errors.

- Help mobile robots drive in a straight line and maintain speed on uneven ground.

STRENGTH:Provide exact linear position measurements down to micro    degrees.

LIMITATIONS:

Some encoders lose their position reference when power is switched off.

Dust, heat, vibration, can affect performance.

- RGB-D Cameras:

## An RGB-D camera captures colour images (RGB) and depth information (D), showing what an object looks like and how far away it is.

USES:

- Recognizes objects and people.

- Estimates object positions for grasping.

STRENGTH:Combines visual information with distance measurements.

LIMITATIONS: Strong sunlight can affect some depth-sensing technologies.

- UltraSonic Sensors: Ultrasonic sensors use high-frequency sound waves to measure the distance to nearby objects.

USES :

- Detects nearby obstacles.

- Measures short-range distances.

STRENGTH: Works in darkness because it uses sound not visible light.

LIMITATIONS : Usually has a shorter effective range than many LiDAR systems.

- Tactile Sensors : Tactile sensors detect physical contact, pressure, and sometimes  forces or vibration. It enablkes safer physical interaction in humanoids.

USES:

- Helps prevent objects from slipping out of the hand.

- Detects contact between the robot and objects.

STRENGTH: Provides Direct feedback about touch and pressure.

LIMITATIONS: Sensors can wear out with repeated Use.

- PCB ( Printed Circuit Board) : A PCB is a board that physically supports and electrically connects electronic components.

USES :

- Connects sensors, processors, and other electronics.

- Supports power distribution and motor-control circuits.

STRENGTH:  Organizes electronic components compactly.

LIMITATIONS : Can be damages by Moisture, Heat.

- DoF(Degree of Freedom) :  It refers to number of independent ways a robot or one of its parts Move.

DoF in a humanoid robot:

- 1 DoF: A simple hinge joint that bends in one direction, such as an elbow with a single rotational axis.

- 3 DoF: A shoulder joint that can rotate about three independent axes.

- Many DoF: A humanoid with independently controlled joints in its neck, arms, hands, waist, and legs.

Why does DoF matter?

- Use: More DoF allows a robot to move more flexibly and perform complex tasks.

- Strength: It helps the robot perform precise movements.

- Limitation: More DoF increases cost, power consumption, and control complexity.

- Actuator in Robotics :

In simple terminology , an actuator converts energy into Physical Movement or Force.

In Robots :

- Moves the robot’s arms legs and fingers.

- Produces the force needed for Walking , lifting , grasping, etc.th

STRENGTH:

- Can provide precise positioning and high force, depending on its design.

LIMITATIONS:

- More DoF can make control more complicated.

- Additional joints can increase cost, weight, and power consumption.

### Types of actuators

| Type | Energy source | Common use |
| --- | --- | --- |
| Electric | Electrical energy | Robot joints, hands, neck, and arms |
| Hydraulic | Pressurized fluid | High-force robotic mechanisms |
| Pneumatic | Compressed air | Grippers and compliant mechanisms |

- Graphics Processing Unit : A GPU is a specialized processor designed to perform many calculations in parallel. Although GPUs were originally developed mainly for graphics, they are now widely used for artificial intelligence, computer vision, and robotics.

Why does a humanoid robot need a GPU?

Humanoid robots may process large amounts of visual and other sensor data in  real time. A GPU can accelerate many of these computational tasks.

- A GPU does not automatically perform all these tasks on its own. Robotics software and suitable AI models must use the GPU to benefit from its parallel-processing capabilities.

USES :

- Processes camera images and other visual data.

- Supports object recognition, depth processing, and scene understanding.

STRENGTH:  Performs many calculations simultaneously.

LIMITATIONS :

- They generate heat and may require cooling.

- Powerful GPUs can increase a humanoid's weight, size, and cost.